# ExtraFax Short Message Peer to Peer (SMPP) Specification

## Protocol Support Level

ExtraFax supports binding to SMSC using SMPP protocol 3.3 and 3.4 only. However, not all PDU types and features are supported.

## PDU Support

The following table shows the PDU supported by ExtraFax

| PDU | Support |
| --- | --- |
| bind_transmitter | Yes |
| bind_transmitter_resp | Yes |
| bind_receiver | Yes |
| bind_receiver_resp | Yes |
| bind_transceiver | Yes |
| bind_transceiver_resp | Yes |
| outbind | No |
| unbind | Yes |
| unbind_resp | Yes |
| submit_sm | Yes |
| submit_sm_resp | Yes |
| submit_sm_multi | No |
| submit_sm_multi_resp | No |
| data_sm | No |
| data_sm_resp | No |
| deliver_sm | Yes |
| deliver_sm_resp | Yes |
| Query_sm | No |
| Query_sm_resp | No |
| cancel_sm | No |
| cancel_sm_resp | No |
| replace_sm | No |
| replace_sm_resp | No |
| enquire_link | Yes |
| enquire_link_resp | Yes |
| alert_notification | No |
| generic_nack | Yes |

## Non-Unicode Character Set Support

When an outgoing SMS does not contain Unicode character, it will be encoded according to the GSM 03.38 character set.

| The GSM 03.38 Default Character Set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dec |  | 0 | 16 | 32 | 48 | 64 | 80 | 96 | 112 |
|  | Hex | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 0 | 0 | @ | Δ | SP | 0 | ¡ | P |  | p |
| 1 | 1 | £ | _ | ! | 1 | A | Q | a | q |
| 2 | 2 | $ | Φ | " | 2 | B | R | b | r |
| 3 | 3 | ¥ | Γ | # | 3 | C | S | c | s |
| 4 | 4 | è | Λ | ¤ | 4 | D | T | d | t |
| 5 | 5 | é | Ω | % | 5 | E | U | e | u |
| 6 | 6 | ù | Π | & | 6 | F | V | f | v |
| 7 | 7 | ì | Ψ | ' | 7 | G | W | g | w |
| 8 | 8 | ò | Σ | ( | 8 | H | X | h | x |
| 9 | 9 | Ç | Θ | ) | 9 | I | Y | i | y |
| 10 | A | LF | Ξ | * | : | J | Z | j | z |
| 11 | B | Ø | <ESC> | + | ; | K | Ä | k | ä |
| 12 | C | ø | Æ | , | < | L | Ö | l | ö |
| 13 | D | CR | æ | - | = | M | Ñ | m | ñ |
| 14 | E | Å |  | . | > | N | Ü | n | ü |
| 15 | F | å | É | / | ? | O | § | o | à |

| Sending additional characters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Original Character |  |  |  | Encoded Character |  |  |
| Character | Decimal | Hex |  | Characters | Hex | Decimal |
| € |  |  |  | <ESC> e | 1B 65 | 27 101 |
| <FF> | 10 | 0C |  | <ESC> <LF> | 1B 0A | 27 10 |
| [ | 91 | 5B |  | <ESC> < | 1B 3C | 27 60 |
| \ | 92 | 5C |  | <ESC> / | 1B 2F | 27 47 |
| ] | 93 | 5D |  | <ESC> > | 1B 3E | 27 62 |
| ^ | 94 | 5E |  | <ESC> ^ | 1B 14 | 27 20 |
| { | 123 | 7B |  | <ESC> ( | 1B 28 | 27 40 |
| ¦ | 124 | 7C |  | <ESC> @ | 1B 40 | 27 64 |
| } | 125 | 7A |  | <ESC> ) | 1B 29 | 27 41 |
| ~ | 126 | 7E |  | <ESC> = | 1B 3D | 27 61 |

Note:

1) Greek characters (Hex 10 – Hex 1A) are not supported.

2) If an outgoing SMS contains any characters not found in the GSM 03.38 character set, the entire content of the SMS will be encoded using Unicode.

## Other features not supported

The following features are not supported:

Delivery Receipt

Delivery Acknowledgement

## Optional Parameters

When binding to SMSC using protocol version 3.4, only the following optional parameters will be handled when deliver_sm is received:

message_payload

sar_msg_ref_num

sar_total_segments

sar_segment_seqnum